Project Name: Voice-Enabled IoT Smart Switchboard

Developed By: Gautam Pal

Date: April 2026

System Status: Functional & Deployment-Ready

## 1. System Overview

The Smart WiFi Switchboard is an advanced IoT-based home automation solution that allows users to control electrical appliances (like Lights and Fans) through a local web interface and voice commands. The system operates as a hybrid device—acting as both a standalone WiFi Access Point and a Station that can connect to the internet for broader access.

## 2. Hardware Architecture

The system is built on a high-performance microcontroller architecture:

- Microcontroller: ESP32 Development Board (Dual-Core, WiFi + Bluetooth)

- Control Unit: 2-Channel Relay Module (Active Low logic)

- Output Pins: * GPIO 26: Connected to Relay 1 (Light)

  - GPIO 27: Connected to Relay 2 (Fan)

- Power Source: 5V DC Power Supply

## 3. Key Features & Functionality

- Dual-Mode Connectivity: * AP Mode: Creates its own WiFi network (Home_Automation_ESP32) for direct local control.

  - STA Mode: Can scan and connect to local home/office WiFi networks.

- Responsive Web Dashboard: A built-in Premium Dark-Theme UI for toggling switches from any smartphone or PC browser.

- AI Voice Control: Integrated Web Speech API allows users to say commands like "Turn on Light" or "Turn off Fan" directly through the web interface.

- Smart WiFi Manager: Includes a real-time WiFi scanner to detect nearby networks and connect dynamically without hardcoding credentials.

- Active Low Protection: Optimized code to ensure relays remain OFF during boot-up to prevent power surges.

## 4. Operational Guide (Step-by-Step)

### Phase 1: Local Connection

- Power On: Plug the ESP32 into a power source.

- Connect WiFi: On your phone, connect to the WiFi named "Home_Automation_ESP32" (Password: 12345678).

- Open Dashboard: Open your browser and go to 192.168.4.1.

### Phase 2: Voice & Manual Control

- Manual: Tap the animated toggle switches on the screen to turn appliances ON/OFF.

- Voice: Tap the "🎤 Tap to Speak" button and give your command in English.

### Phase 3: Internet Setup

- Click on "📡 Connect to Internet".

- Select your home WiFi from the list.

- Enter your WiFi password and click Connect. The device will now bridge to your local network.

## 5. Software Technical Stack

- Language: C++ (Arduino Framework)

- Libraries: WiFi.h, WebServer.h

- Frontend: HTML5, CSS3 (Flexbox), JavaScript (Fetch API, Web Speech API)

- Communication: Asynchronous HTTP Requests for lag-free switching.